EMAS ELEKTROTEKNİK MAKİNA    SANAYİ VE TİCARET A.Ş

Ziya Gökalp Mah. İkitelli Organize Sanayi Bölgesi Atatürk Bulvarı No:50 34490 Başakşehir / İstanbul / TÜRKİYE

Telefon : +90 212 549 22 52

Faks      : +90 212 549 25 80

Web      : www.emasas.com.tr

Technical Specifications : B100DB

Order Code:

B100DB

Contact Block:

1 NO

Terminals with finger protection.

Fireproof plastic contact block body.

Sequential connectable body and contact blocks after lego magic.

Protective cable inlet tips enabling maximum safety.

Screw Washers.

Technical Information :

| Body: |  |
| --- | --- |
| Plastic Body. |  |
| Opportunity of positioning with plates and Badge Bases. |  |
| General Specifications |  |
| Operating Temparature | -5/+40°C |
| Mechanical life | 0,5 Million (3000 open-close/hour |
| Electrical Life | 0,1 Million (1800 open-close/hour) |
| C.Block Tightening Torque | 0,6-0,8 Nm |
| C.Block Connection Wire Profile | Min: 1x0,5 mm'2 |
| Wire Profile | Max: 2x2,5 mm'2 |
| Protection Degree | IP50 |

Electrical Informations :

| General Specifications |  |  |
| --- | --- | --- |
| Range Of Use |  | AC15 |
| Operating Voltage | Ue | 230V AC |
| Fregquency | F | 50Hz |
| Operating Current | Ie | 4A |
| Insulation Resistance | Ui | 300V |
| Stroke Resistance Voltage | Uimp | 2.5kV |
| Dielectrical Strength |  | 1500V AC |

| General Specifications |  |  |
| --- | --- | --- |
| Range Of Use |  | AC15 |
| Operating Voltage | Ue | 250V AC |
| Fregquency | f | 50Hz |
| Operating Current | Ie | 4A |
| Insulation Resistance | Ui | 300V |
| Stroke Resistance Voltage | Uimp | 2.5kV |
| Dielectrical Strength |  | 1500V AC |

Electrical Informations :

Standards:

- TS EN 60947 -5-1

In contact Blocks;

- VDE 0660

-  UL 508